G-NOR ELECTRONICS CO.,LTD.

| # GNL-1206xx (1206 SMD TYPE) Ultra Brightness ABSOLUTE MAXIMUM RATING:(Ta=25℃) Parameter; Unit. DC Forward Current; 30; mA. Reverse Voltage; 5; V. Reverse Current (Vr-5V); 10; µA. Peak Forward Current ,tw ,=1 msec Duty ,=1/20; 150; mA. Operation Temperature; -40; +80; ℃. Storage Temperature; -40; +100; ℃. Solder Dip Temperature (5 seconds,1.6mm from body ); 300; ±5; ℃. |  |
| --- | --- |
| Features: Compatible with automatic placement equipment Compatible with reflow solder process Applications: Automotive and Telecommunication General use for indicators |  |
| Part Number; EMITTED COLOR; Lens Color; Peak Wavelength (nm); Vf (v) Typ.; IV (If=20mA) mcd; View Angle 2θ1/2. Min.; Typ.. GNL-1206GC; GaP; Green; Water Clear; 567; 2.0; 7; 12; 120°. GNL-1206EC; GaP; Orange Red; Water Clear; 635; 2.0; 10; 18; 120°. GNL-1206YC; GaAsP; Yellow; Water Clear; 588; 2.0; 10; 18; 120°. GNL-1206SRC; GaAlAs; Red; Water Clear; 660; 1.9; 9; 15; 120°. GNL-1206URC; GaAlAs; Super Red; Water Clear; 660; 1.9; 65; 95; 120°. GNL-1206HRC; AlGaInP; Ultra Red; Water Clear; 640; 2.1; 30; 40; 120°. GNL-1206UYC; AlGaInP; Super Yellow; Water Clear; 587; 2.1; 30; 40; 120°. GNL-1206UBC; InGaN; Super Blue; Water Clear; 470; 3.2; 40; 50; 120°. GNL-1206PGC; InGaN; Super Pure Green; Water Clear; 525; 3.4; 70; 120; 120°. GNL-1206WC; InGaN; Super White; Water Clear; 3.2; 60; 100; 120°. |