G-NOR ELECTRONICS CO.,LTD.

| # GNL-0603xx (0603 SMD TYPE) Ultra Brightness ABSOLUTE MAXIMUM RATING:(Ta=25℃) Parameter; Unit. DC Forward Current; 30; mA. Reverse Voltage; 5; V. Reverse Current (Vr-5V); 10; µA. Peak Forward Current ,tw ,=1 msec Duty ,=1/20; 150; mA. Operation Temperature; -40; +80; ℃. Storage Temperature; -40; +100; ℃. Solder Dip Temperature (5 seconds,1.6mm from body ); 300; ±5; ℃. | Unit: mm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Features: Compatible with automatic placement equipment Compatible with reflow solder process Applications: Automotive and Telecommunication General use for indicators |  |  |  |  |  |  |  |  |  |
| ## Part Number; EMITTED COLOR; Lens Color; Peak Wavelength (nm); Vf (v) Typ.; IV (If=20mA) mcd; View Angle 2θ1/2. Min.; Typ.. GNL-0603GC; GaP; Green; Water Clear; 567; 2.0; 7; 12; 120°. GNL-0603EC; GaP; Orange Red; Water Clear; 635; 2.0; 10; 18; 120°. GNL-0603YC; GaAsP; Yellow; Water Clear; 588; 2.0; 10; 18; 120°. GNL-0603SRC; GaAlAs; Red; Water Clear; 660; 2.0; 9; 15; 120°. GNL-0603LRC; GaAlAs; Super Red; Water Clear; 660; 1.9; 30; 40; 120°. GNL-0603UGC; AlGaInP; Yellow green; Water Clear; 572; 2.0; 22; 35; 120°. GNL-0603UYC; AlGaInP; Super Yellow; Water Clear; 587; 2.1; 30; 40; 120°. GNL-0603UGC; AlGaInP; Yellow Green; Water Clear; 572; 2.1; 25; 35; 120°. GNL-0603HRC; AlGaInP; Super Red; Water Clear; 640; 2.1; 30; 40; 120°. GNL-0603UYOC; AlGaInP; Super Amber; Water Clear; 605; 2.1; 30; 40; 120°. GNL-0603UBC; InGaN; Super Blue; Water Clear; 470; 3.2; 40; 100; 120°. GNL-0603PGC; InGaN; Super Pure Green; Water Clear; 525; 3.4; 200; 400; 120°. GNL-0603WC; InGaN; Super White; Water Clear; 3.2; 60; 100; 120°. GNL-0603UWC; InGaN; Super White; Water Clear; 3.2; 100; 150; 120°. |
| GNL-0603UYC-TL | AlGaInP | Super Yellow | Water Clear |  | 590 | 2.1 | 60 | 150 | 120° |
| GNL-0603UHRC-TL | AlGaInP | Hyper Red | Water Clear |  | 630 | 2.1 | 100 | 200 | 120° |